📌 README.md (Final Polished Version)

Minimal Camera Network Optimization

MATLAB implementation of the ILP-based minimal camera network optimization framework proposed in "Autonomous BIM-Aware UAV Path Planning for Construction Inspection". Includes visibility simulation, penalty matrices, ILP solver (intlinprog), and example datasets for UAV inspection scenarios. Minimal Camera Network Optimization

MATLAB implementation accompanying the paper: “Autonomous BIM-Aware UAV Path Planning for Construction Inspection” (B. Alsadik, 2025)

🔍 Overview

This repository provides the full MATLAB implementation of the ILP-based minimal camera network optimization framework described in the paper.
The method takes a dense set of candidate viewpoints around a structure, computes visibility and photogrammetric penalty matrices (GSD, B/H, triangulation accuracy), and solves an Integer Linear Program to select the smallest subset of cameras that still satisfies coverage and quality constraints.

The framework is designed for UAV-based inspection and photogrammetry, enabling significant reductions in:

- number of images

- mission duration

- battery consumption

- redundancy in coverage

…while preserving reconstruction accuracy.

🗂 Repository Structure

Minimal-camera-network/

│

├─ src/

│   ├─ ilp_camera_optimization_with_accuracy.m

│   ├─ visibilitytesting.m

│   ├─ visibilitytesting_plotting.m

│   ├─ read_wpk.m

│

├─ examples/

│   ├─ run_minimal_ILP_demo.m

│   ├─ example_wpk.mat

│   ├─ example_points.mat

│

├─ README.md

├─ LICENSE

└─ .gitignore

- src/ holds the core ILP and visibility functions.

- examples/ provides a minimal dataset and a ready-to-run demo script.

🚀 Quick Start

1. Clone the repository

git clone https://github.com/Photogrammetry-Topics/Minimal-camera-network.git

cd Minimal-camera-network

2. Open MATLAB and add the repo to your path

addpath('src');

addpath('examples');

3. Run the demo

run_minimal_ILP_demo

This script:

- Loads a minimal example dataset

- Compute visibility between cameras and 3D points

- Generates penalty matrices (GSD, B/H, intersection angle)

- Runs the ILP using MATLAB’s intlinprog

You should see output like:

Selected 72 / 180 candidate cameras in x sec

🧠 Method Summary

The approach consists of the following steps:

- Generate candidate viewpoints
Using standard photogrammetric sampling patterns (double-grid, circular belts, oblique rings).

- Compute visibility
For each camera–point pair using visibilitytesting.m.

- Evaluate photogrammetric quality

  - Ground Sampling Distance (GSD)

  - Base-to-Height ratio (B/H)

  - Intersection-angle accuracy

- Formulate ILP
Select the minimal number of cameras while satisfying:

  - Visibility coverage

  - Quality constraints

  - Optional penalties for redundancy

- (Optional) Plan UAV path
The selected cameras can serve as waypoints for TSP-based or RRT-based flight planning.

📁 File Descriptions

ilp_camera_optimization_with_accuracy.m

Core ILP solver based on MATLAB’s intlinprog. Implements the objective function and all photogrammetric constraints defined in the paper.

visibilitytesting.m

Ray-based visibility simulation between candidate camera poses and 3D scene points.

visibilitytesting_plotting.m

Utility for visualizing dense and optimized camera networks.

read_wpk.m

Helper function to load precomputed camera penalties (orientation vectors, weights, etc.).

run_minimal_ILP_demo.m

End-to-end example script demonstrating the full pipeline on a lightweight dataset.

🧩 Dependencies

- MATLAB R2024a or newer

- Optimization Toolbox (intlinprog)

- No external solvers required (no Gurobi or CPLEX)

📄 Citation

If you use this code, please cite:

Abdulateef, N. A.; Jasim, Z. N.; Hasan, H. A.; Alsadik, B.; Khalaf, Y. H. Autonomous BIM-Aware UAV Path Planning for Construction Inspection. Preprints 2025, 2025102437. https://doi.org/10.20944/preprints202510.2437.v1

BibTeX will be included once the paper is published.

🌐 License

This project is released under the MIT License, allowing academic and commercial reuse with attribution.